NOTES FOR GUIDANCE ON FIRE SAFETY AT PUBLIC EVENTS

INTRODUCTION

This guidance note set out the important fire safety points to be considered, when any premises including; tents, marquees or other temporary structures that are used for public events.

It should be noted that those responsible for arranging events may need to apply through relevant legislation to Licensing Authorities e.g. Licensing Act 2003.

Irrespective of any type of application required, it is likely that the event will be covered by primary fire safety legislation and cognisance needs to be taken of these requirements.

GENERAL STATEMENT

It is vital that all venues to which the public are admitted are as safe as possible from fire and that the organisers know how to protect the public and staff from injury or death in any emergency situation. From a fire safety point of view there will be instances in which venues may not be suitable for the intended purpose and as such additional safety measures may be required to ensure the safety of those at the event.

This note is designed to assist the organisers and applicants in highlighting key areas that may need to be addresses to further ensure event safety. It should be noted that it cannot cater for all different circumstances and if there are any doubts about fire safety those responsible should seek further advice and information as appropriate.

Sources of reference are provided at the end of the note.

RISK ASSESSMENT

A fire risk assessment helps you systematically identify all the fire hazards and risks that the event may present.

It involves identifying the hazards present (whether arising from work activities or from other risks

e.g. the layout of the venue), evaluating the extent of the risks, taking into account whatever precautions have already been taken and the implementation of control measures.

There are six steps, which need to be taken to assess the risk associated with staging the event:

- Identify the hazards associated with activities contributing to the event, where the event is carried   	out and how the activities are to be undertaken;

- Identify those exposed to the hazards;

- Evaluate the risks, taking account of existing precautions (this will indicate the level of residual 	risk);

- Devise control measures, which reduce the residual risks to a level as low as reasonably 	practicable;

- Implement the control measures; and

- Monitor, review, and revise the assessment as necessary to maintain its effectiveness.

There are many options available to help applicants to carry out their fire risk assessment.

One option is to get a Fire Risk Assessment Consultant to do your fire risk assessment for you or alternatively, if you decide to carry out your own fire risk assessment, there are a number of tools to help you identify and evaluate the risks. Information relating to these aspects may be found online, at local information sources or stationary stores.

MEANS OF ESCAPE

The structure should be provided with sufficient exits for public use in an emergency. Structures holding 50 or more persons should have at least 2 exits suitably positioned to provide a true alternative.

The number of exits should be capable of evacuating the occupancy in 2 minutes or less, in any given fire/emergency situation. They should be distributed around the structure.

EXITS

Exits should be provided by the use of frames and doors which:

| i. | Are firmly secured in place and do not rely on the canvas of a tent to provide support. |
| --- | --- |
| ii. | Open in the direction of egress. |
| iii. | Are fitted with an approved panic-type fastening. |

Alternatively, leaving out or fastening back sections of a sidewall may form exits. (These must remain open whilst ever the structure is occupied.)

Clear exit width must be maintained and continued to a final place of safety, i.e. away from the premises/ structure

Normally no exit should provide less than 1.05 metres clear opening.

Final exits should be distributed evenly around the structure so that the distance of travel is not more than 18 metres.

Note

In rooms with one exit the maximum distance of travel should normally be no more than 6.5 metres.

Exit routes should be kept clear at all times.

GENERAL LIGHTING

Adequate lighting should be provided both inside and outside the structure if it is to be occupied during the hours of darkness. It should remain on until all members of the public have left the site.

Lighting should be by electricity and comply with safety requirements laid down by IEE regulations for electrical equipment buildings (temporary installations). It should, as far as possible, be clear of canvas and be positioned where it cannot be tampered with.

The use of candles/ naked flames is not recommended.

EMERGENCY LIGHTING

An emergency lighting system by means of self-contained units which will illuminate the full extent of all escape routes and exit/emergency exit directional signs will be considered suitable.

These units should be so connected that illumination will be provided on the failure of the general lighting circuits.

COOKING FACILITIES

Where cooking facilities are provided they should be at least 6 metres from the main marquee (if applicable). This may be linked by a covered walkway.

Where gas is to be provided from LPG cylinders, the following precautions should be taken:

| i. | Gas is to be piped to cooking area. |
| --- | --- |
| ii. | Cylinders should be secured in open air and protected from unauthorised interference and |
|  | accidental spillage. |
| iii. | Screw connections should be tightened and rubber hoses secured with an appropriate |
|  | fastening device. |
| iv. | Containers should be marked to indicate their contents. |

FIRE EXTINGUISHERS

Sufficient fire extinguishers should be available in the following staff areas:

i.	9-litre water type extinguishers behind bar serveries and at each manned exit.

ii.	Carbon dioxide extinguishers by all stage areas and DJ consoles.

iii.	4.5 kilos dry powder extinguisher in all kitchen areas.

iv.	Fire blanket in all kitchen areas.

v.	Dry powder/carbon dioxide extinguishers at all LPG installations and generator 	installations.

All portable fire extinguishers should have the following examinations:

i.	A regular inspection by the user to make sure that appliances are in their correct position and have not been discharged or lost pressure (extinguishers fitted with pressure indicator).

ii.	An annual inspection, service and maintenance by a BAFE (British Approvals of Fire Equipment) registered engineer, in accordance with clauses 8 and 9 of current British Standard 5306 Part 3.

Records should be kept of all examinations and tests of portable fire extinguishers.

SOURCES OF REFERENCE

HMSO PUBLICATIONS

Guide to Fire Precautions in Existing Places of Entertainment and Like Premises Guide to Health Safety and Welfare at Pop Concerts and Similar Events

AVAILABLE FROM AUTUMN 2006